Data suppression firmware for ALPHA

The ALPHA capture buffer (ALPHAlatency.bdf):

This is a modified version of the normal VF48 latency buffer. Its purpose is to capture one complete ASIC readout sequence. The capture is initiated by the external trigger. Then, the input data is sampled at 40 MS/sec. Externally, the data is shifted out of the ASIC at a frequency = 40 MHz/N, where N is a parameter equal to 20 if the readout is performed at 2 MHz.  The capture process stores the digitized information in a memory for only one out of N FADC samples; and does so for a period lasting 128 memory write cycles. In this example, the capture requires 128*N*25 = 64 microseconds. When the buffer is completely filled, the capture buffer logic issues a “transfer_command”, a “start” pulse,and a block of data including the current buffer memory address “Radd”. The data is 80-bit wide (8 channels at a time), and the readout is performed at 40 MHz. This block of data, containing the information on 128 X 8 ASIC channels, is then moved to the next buffer stage (ALPHAWindowBuffer.bdf). This operation requires only 128*25 = 3.2 microseconds. Even if the data is not processed yet, the system can be rearmed and be ready to accept another trigger.

Note: The value of N is transmitted by MbitsII [15..8], parameter # 11; but it should be programmed with a value = (N-1) if one wants to divide by N.

Resource: 3 memory blocks

The ALPHA window buffer (AlphaTwindowFIFO.bdf)

This buffer holds one complete event pushed in by the capture buffer. Its purpose is to accept the event with 8 channels in parallel at the input, and output the content one channel at the time towards the zero suppress module in order to compare their value to a pedestal. It first outputs the 128 samples from channel 0, then the 128 samples form channel 1, etc. The single elements are taken out of the buffer by requests from the zero-suppress stage (“ReadNextElement” line). The module also issues flags: “ElementReady”, “SegmentStart”, “SegmentEnd”, SegmentGate”, a segment being one block of 128 samples from the same channel.

This transfer operation is done without interruption is there is enough free buffer space in the next stage. So, the time required is 1024*25 = 25.6 microseconds.

New data suppression firmware module (ZeroSuppress.bdf):

The data suppression mechanism is initiated when the TWINDOW buffers become not empty. This process can be done synchronously at 40 MHZ as soon as one element is detected in the TWINDOW buffer (it fills at the same rate as it is emptied). This mechanism reads the TWINDOW buffer in 8 passes of 128 elements each, one pass for each input port connected to the FPGA. In each pass, the 128 elements of one of the 8 data ports are inspected. The content of each element is compared to the value of the corresponding element of the threshold memory. If it is above threshold, a 24-bit data word is constructed:

Bits 23 down to  22 : 0

21 down to 19 : Port ID ( 0,1, … 7)

18 down to 12 : Sample number (0,1….127)

11 down to 10 : 0

9 down to 0   : Sample amplitude (FADC)

This data word is then pushed into the ZeroSuppressed data buffer.

The data buffer is large enough to hold 4 events with 100% occupancy, or more events if the occupancy is less.  The zero suppressed data buffer FIFO is 4096 X 24 words deep.

Resource: 32 memory blocks, with 16 recovered from the original TWINDOW buffer

Time required to zero suppress one event:    26 microseconds.

NOTE: 26 microseconds is much less than the 50 microseconds required to read the ASIC at 2.5 MHz. or 64 microseconds at 2 MHz.

Thresholds memory (inside ZeroSuppress.bdf):

In each of the 6 VF48 Front End FPGAs, and for each of the 128 elements of a channel readout, a 10-bit signed integer threshold value is stored in a threshold ram. So, there is one value of threshold for each ASIC sample.

This RAM is loaded by the VF48 parameter mechanism.

Parameter ID  0x14 refers to  the address counter of the RAM memory.

Parameter ID  0x15 refers to a read or write operation to the content of that address, followed by a post increment of the address counter. The RAM is organized as a 1024 X 10 bits memory to store 8 X 128 10-bit threshold values.

Resource required in each FE FPGA for 8 X 128 X 12bits : 4 memory blocks.